XLR8 2026 · Initiative Proposal Electronics & Robotics Club, IIT Bombay

| Proposal Title | Live Telemetry Display & Real-Time Leaderboard |
| --- | --- |
| Category | Spectator Experience / Event Infrastructure |
| Target Event | XLR8 2026 — All Rounds |

The Idea

Watching XLR8 right now is a bit like watching a race where you can only see the finish line. You stare at a bot, it crosses, someone reads out a time. The crowd doesn't really know what's happening until it's over.

Here's what I think would fix that: attach a small ESP32 (≈₹220) to each competing bot. It broadcasts live data — speed, which obstacles were cleared, elapsed time — over WiFi to a Raspberry Pi running a simple dashboard on the projector. Every run becomes something the whole room can actually follow in real time.

The key thing is what this does to the crowd. When the leaderboard updates live, people start caring about whether this team is going to beat the current leader before the bot even finishes the course. Non-technical spectators — friends, juniors, parents at open days — can read a number going up and feel the tension. That doesn't happen right now.

What the Display Shows

- Current speed (cm/s) — computed from wheel encoder or IMU, shown large

- Obstacle clear counter — increments live as each sensor detects a clear

- Elapsed run time — takes up half the screen, visible from the back of the room

- Live leaderboard — all completed runs ranked, updates the moment each run ends

- "On pace to beat leader" indicator — flashes if the current run is ahead of the record

How It Actually Works

Bot side →  Each team adds an ESP32 to their bot before the event. ERC provides a standard firmware library — one function call to log each event. Takes under 30 mins to integrate.

Server side →  A Raspberry Pi (ERC already owns several) runs a lightweight Node.js server. It receives UDP packets from each bot and serves a live dashboard at a local IP address.

Display side →  Any laptop browser pointed at the server IP. No internet required — everything runs on a local WiFi hotspot. One projector, done.

Why It Makes XLR8 Better

The honest version: the current format is good at testing whether bots work. This makes it good at being watched. Those are different things, and right now XLR8 only does one of them well.

The other thing I'd point out is reuse. The dashboard, the firmware library, the server setup — all of it works for any future ERC event. You build it once for XLR8 2026 and it's useful for workshops, demos, next year's competition. That's not true of most one-off additions.

Total extra cost for 20 teams: ₹4,400 in ESP32 modules plus one router (≈₹800). The Raspberry Pi and projector are already available.

ERC IIT Bombay  ·  XLR8 Initiative Proposal  ·  2026